VINCI 
MECHANICAL CODE LOCK WITH SECURED ENTRANCE

SPECIFICATIONS

The mechanical code lock shall be 100% mechanical and shall not require any batteries or electricity to function.

The mechanical code lock shall have a solid stainless steel deadbolt with a throw of minimum 23mm (7/8”) to guarantee a secure locking.

The mechanical code lock shall, without any adjustments, be possible to install on square, round or flat profiles.

The mechanical code lock shall allow a key operated self-latching of the bolt.

An anodised aluminium handle pair shall be standard included.

It shall be possible to install different handle pairs on the mechanical code lock.

The mechanical code lock shall have two mechanical code panels on each side.

The mechanical code lock shall allow for entrance and exit with different codes.

The mechanical code lock shall have a polyamide dust and rain cover.

The mechanical code lock shall have an adjustable rollerbolt for easy closing.

The mechanical code lock shall allow an automatic code reset after pushing the handle.

The mechanical code lock shall have an anti-code detection thanks to an integrated security press plate.

The mechanical code lock shall be available in Silver or RAL 6005, 6009, 7016, 9005 and 9010.

The mechanical code lock should be designed and manufactured in Europe or in the United States.

PERFORMANCE

The mechanical code lock shall be specifically made for outdoor use.

The mechanical code lock shall be maintenance free (no greasing).

There shall be no recognizable wear and tear to those push buttons which are frequently used.

The mechanical code lock shall have been tested for 500.000 movements.

The manufacturer warranty shall be 3 years.

INSTALLATION

The mechanical code lock shall be installed onto a 4-hole pattern with 2 hexagon socket head screws.

The mechanical code lock shall allow an easy and quick configuration or modification of the codes.

The self-latching daybolt of the mechanical code lock shall be able to be used for both left and right turning gates.

The mechanical code lock shall allow a 20mm (3/4”) continuous adjustability of the bolts for a perfect alignment of the locking mechanism.

An installation explanatory video shall be available to assist the installer.

An optional drilling jig (Drill-Fix) should be recommended for a correct installation.

MATERIAL

The lockbox of the mechanical code lock shall be KTL / E-coated and the complete code lock shall have a corrosion resistance of 500h salt spray according to ISO9227.

The mechanical code lock shall be made of a vandal-proof, powder coated aluminium housing (no wet painting or anodization). The powder coating shall be according to the Qualicoat standards.

The mechanical code lock shall have a stainless steel mechanism.

The mechanical code lock shall have stainless steel push buttons.

The mechanical code lock shall be fully weather- and dustproof.

The mechanical code lock shall have an UV-resistance against discoloration of 500h.